User    Manual

Please read this manual carefully before use!

Thank you for purchasing this product. To optimize the performance of this product,prior to use,please read these operating instructions carefully to familiarise yourself with the basic operation of this unit.Please keep the manual in safe price for future reference.

This unit has been tested at our factory before being shipped to you, there is no assembly required.

Once after received this product , please take and put carefully. And check cautiously that whether the product was damaged or not during the transportation.

- Checking Parts

Please check if the following parts are included:

1 x LED Light

1 x User manual

1 x Power plug

Caution!

Keep this equipment away from rain,moisture and liquids.

- Installation

- 1. Please check the voltage whether is the same with the one showed on the equipment or not.

- 2. It must ask for the technical person and set the light safety when installation. And let the light beam at the suitable angle.

- 3. When install this equipment please make sure there's no flammable surfaces (decorated things, etc) within at least 2.5M and maintain minimum distance of 0.5M from the equipment to the walls.

- 4. Please make sure that there's no other equipment or decorating materials obstructed the exhaust fan and the vent-pipe.

- 5. Products should be install immobility.

6. In case of safety, it's very important that to connect the earth with line.

Attention

1.This product is A type of LED

2.Please do not open the bottom cover yourself without permission.Operate it accord the user manual. Call the technician when the machine breaks down.

3.Please make sure that this product is installed by professional.

4.Please don’t operate this product in wet or dusty environment.

5.Please don’t touch the product or wire when your hand is wet.

6.Please don’t switch this product continually, and please try to avoid the product working long time.

7.Please fix this product in a certain place, so that to avoid it from violent strike.

8.Please avoid foreign bodies going into the machine.

9.The distance between the lamp and the object should be at least 1 meter.

10.Please don’t connect the power or turn on the lamp before installation.

11.Please turn the brightness of LED to be the lowest before connecting or disconnecting the power.

12.Please check the plug and power cord before connecting the power.

13.Please turn off the lamp for at least 25 minutes if the lamp has been turned on for 3 hours.

14.Please use the original package and take the lamp gently when transporting the product.

Display Menu:

| 1 | d001 | 001- 512 | DMX512 Address code setting -4 channel mode |
| --- | --- | --- | --- |
| 2 | A001 | 001- 512 | DMX512 Address code setting -8 channel mode |
| 3 | U255 | 000 - 255 | Cold white dials, from dark to bright. |
| 4 | C255 | 000 - 255 | Warm white dials from dark to bright. |
| 5 | FH99 | 01 - 99 | Stroboscopic, from slow to fast. |
| 6 | CC99 | 01 - 99 | Jump, from slow to fast. |
| 7 | DE99 | 01 - 99 | Gradient, from slow to fast. |
| 8 | CP99 | 01 - 99 | Pulse change, from slow to fast. |
| 9 | S0UD | - | saltus step |

Channel Instruction

8channel

| Channel | Function | Description |
| --- | --- | --- |
| CH1 | Dimming | U/V dimming，From dark to bright |
| CH2 | U1 dimming | U1 dimming，From dark to bright |
| CH3 | V1 dimming | V1 dimming，From dark to bright |
| CH4 | U2 dimming | U2 dimming，From dark to bright |
| CH5 | V2 dimming | V2 dimming，From dark to bright |
| CH6 | Total Strobe | U/V total strobe, slow to fast |
| CH7 | function | 0-50:DMX dimming; 51-100: common color output (8); 101-150: jump; 151-200: gradual; 201-250 pulse; 251-255:sound control |
| CH8 | Function speed | Function speed, from slow to fast |

4channel

| Channel | Function | Description |
| --- | --- | --- |
| CH1 | Dimming | U/V dimming，From dark to bright |
| CH2 | U1 dimming | U1 dimming，From dark to bright |
| CH3 | V1 dimming | V1 dimming，From dark to bright |
| CH4 | U2 dimming | U2 dimming，From dark to bright |

- Technical Specifications

1. Power supply: 100~240VAC,50~60Hz

2. Rated power: 200W

3. Lamp spec: 2*100W

4. Control signal: DMX512 signal

5. Control mode: Sound Active/ Auto-play/DMX512/Master-Slave

6. Channel: 4/8

7.Power connector: PoweCon input/output

9.Color temperature:3000-5600k

10.Beam angle:60degrees

11. Illumination (Lux):

- WW full bright:3.95lux@1M, CW full bright:3.50lux@1M, WW and CW full bright:5.76lux@1M

- WW full bright:1.2lux@1M, CW full bright:0.95lux@1M, WW and CW full bright:1.70lux@1M

12.Weight:2.5kg

13.Dimensions.:340*150*130mm

Declaration

This device has left our premises in absolutely perfect condition. In order to maintain this condition and to ensure a safe operation, it is absolutely necessary for the user to follow the safety instructions and warning notes written in this user manual.

We cannot be made liable for damages caused by incorrect installations and unskilled operation!

Warranty Card

This product is made of high-brightness transistor. We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product need maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：