LED Pixel Par Light

User Manual

Please keep the manual carefully for future reference

Table of Contents

Introduction--------------------------------------------------------------------------P2

Symbol instruction on the device----------------------------------------------P3

Safety and description of device-----------------------------------------------P4

Menu operation instruction-----------------------------------------------------P5

DMX512	Channels list----------------------------------------------------------- P6

DMX512 connection/DMX address setting-----------------------------------P9

Warranty card/Maintenance-----------------------------------------------------P10

Introduction

Thank you using our LED series products. This fixture is the latest new LED product.The product housing used plastic material, beautiful appearance, with international advanced electronic control technology and excellent industrial design. Product effect more rich, colorful, and color is gorgeous because of installed high output light source.

User can invoke the built-in program, scenario editors and master-slave control mode setting by menu. Also can setting rainbow, flash frequency and electric dimmer effect.

This product has a fast response, seismic, high impact resistance, good heat dissipation, high protection grade, long using life(LED lift is 100,000hrs), no radiation, high efficiency and energy saving, how energy consumption, really the green products. Fully accepted international standard DMX512 signal.

This product applications widely, installation simple. Can widely used for the large fixed stage, lighting background dyeing, large-scale theatrical performances, stadium lighting, temporary stage, urban lighting system, TV, conference center, professional theatre, park, dance hall, bars, small artistic performances and public lighting environment.

Packing Include

● 1* Power cable

● 1* User manual

- Caution

- Before out factory, this device passed strict inspection, functions stable and perfect packing. If this   device will be operated in any way different to the one described in this manual, the product may suffer  damages and the guarantee becomes void. Furthermore, any other operation may lead to dangers like short circuit, burns, electric shock, crash etc. If any technology changes of this manual, company do not notice in further and have interpretation.

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Symbol Instruction On The Device

-------------------------------------------------------------------------------CE certificate

---------------------------------------------------------------------Only indoor using

------------------------------------------------------------------------------RoHS certificate

----------------------------------------The minimum distance from lighted objects(meter)

--------------------------------------------------------------Max ambient Temp

-----------------------------------------------------------Max housing Temp

------------------------------------------------------------------------- Safety caution

--------------------------------------------------------------------Be careful of burn

---------------------------------------------------------------------Danger high voltage

---Not allow installation directly on the surface of ordinary combustible materials

- Safety instruction

- Once after received this product , please take and put carefully. And check cautiously that whether the product was damaged or not during the transportation, if yes, please not use this fixture and contact local dealers or manufacturers as soon as possible.

- The applicable temperature for the lighting is below 40°C. Do not use the lighting above the temperature.ta=40℃

- The fixture applicable widely, it must keeping clean, Please do not in overheat or dusty environment  use the device. Do not touching Chemical liquid.

- Should be paid attention to the use of this product: fire, heat, electric shock, loss caused by severe or fatal injury, in electricity or to read the instructions before installation, follow the safety instruction and pay attention to the manual and warning signs on the equipment, if you have any other questions about how safe operating equipment, please contact the dealer or manufacturer as soon as possible.

- According to the procedures described in this operating instructions, please do not open by yourself. Any break, please contact professionals, and must by the professional and technical personnel for repair.

- Do not power in and open before installation. Let have a rest when continuous work for eight hours. It will longer the device using lift.

Fixed installation, to prevent the strong vibration or impact of the device.

- The distance between the device and the projectile to keep at least 0.5 meter.

- Never look directly into the light source,as sensitive persons may suffer an epileptic shock(especially meant for epileptics)!

- When light output position of the device of dustproof glass breakage, cracks or other visible damage, user should replace the new glass to continue to use.

- Installation

- When install this equipment please make sure there's no flammable surfaces (decorated things, etc) and minimum distance of 0.5M from the equipment to the flammable.

- The installation of the effect has to be built and constructed in a way that it can hold 10 times the weight and without any harming deformation.

- The installation must always be secured with a secondary safety attachment, e.g. an appropriate safety rope.Never stand directly below the device when mounting, removing or servicing the fixture.

三、Product Specification

- 3/8 DMX512 channels mode

- Light source: high power LED, long use life, high output, no radiation and low consumption.

- DMX/Auto/Sound/Master-slave control mode.

- LED using static driving mode of constant current, constant current high precision level 256 brightness, flicker-free.A variety of strobe effect.

- LED display

- Built-in different inner program, can directly invoking it.

- 3-pins XLR

- Power supply：AC100V-240V  50/60Hz； Rated power：80W

- 5050 Patch LED：325 X 0.2W RGB LED

- Max ambient Temp：40℃.

- Housing max ambient Temp：65℃.

- Product size:313 X 293 X 117mm

LED digit display menu:

MENU     UP   DOWN       ENTER         D

LED digit display menu instruction

| A001 |  | A512 | Address setting A001~A512 (At 8CH mode setting) |
| --- | --- | --- | --- |
| D001 |  | D512 | Address setting A001~A512 (At 3CH mode setting) |
| C001 |  |  | No Function |
| P001 |  | P115 | Inner patterns(P001-P115) |
| CF00 |  | CF07 | Inner patterns color（CF00~CF07） |
| SP01 |  | SP16 | Inner patterns speed（SP01~SP16） |
| ST00 |  | ST63 | Strobe speed（ST00~ST63） |
| Soud |  | Soud | Sound |
| R255 |  | R000 | Red dimmer（R000~R255） |
| G255 |  | G000 | Green dimmer（G000~G255） |
| B255 |  | B000 | Blue dimmer（B000~B255） |
| T000 |  |  | Temp display, if show T045 means the temp of fixture is 45℃ at the moment; It will showT000 if without installed 10Kthermistor |

Factory setting

At the address data A001-A512, press MENU with 5 sec enter into Factory setting. Here mainly setting fixture’s power output, LEDs quantity, temp protection, sending specification. After finished setting, press Enter 5 sec exit.

| R032 |  | R255 | Adjust Red current（R032-R255） |
| --- | --- | --- | --- |
| G032 |  | G255 | Adjust Green current（G032-G255） |
| B032 |  | B255 | Adjust Blue current（B032-B255） |
| S325 |  |  | Cannot modify, fixture’s LEDs total are 325pcs |
| T040 |  | T105 | Setting fixture temp protection data（40℃~105℃） |
| Send |  | Send | Send this unit factory spec to anothers which all connected this unit. Press 5 sec and exit confirm to send |

DMX512

8 channels mode instruction

| Channel | Data | Function |
| --- | --- | --- |
| 1 | 000-255 | Total dimmer |
| 2 | 000-255 | Red dimmer |
| 3 | 000-255 | Green dimmer |
| 4 | 000-255 | Blue dimmer |
| 5 | 000-255 | Strobe |
| 6 | 000-002 | No function |
|  | 003-005 | Color option |
|  | 006-008 | Gradual |
|  | 009-011 | Jump |
|  | 012-014 | Pulse |
|  | 015-017 | Wave gradual |
|  | 018-020 | R-G-B Circle gradual |
|  | 021-023 | R-G-B Circle jump |
|  | 024-026 | Effect 1 |
|  | 027-029 | Effect 2 |
|  | 030-032 | Effect 3 |
|  | 033-035 | Effect 4 |
|  | 036-038 | Effect 5 |
|  | 039-041 | Effect 6 |
|  | 042-044 | Effect 7 |
|  | 045-047 | Effect 8 |
|  | 048-050 | Effect 9 |
|  | 051-053 | Effect 10 |
|  | 054-056 | Effect 11 |
|  | 057-059 | Effect 12 |
|  | 060-062 | Effect 1 3 |
|  | 063-065 | Effect 1 4 |
|  | 066-068 | Effect 1 5 |
|  | 069-071 | Effect 1 6 |
|  | 072-073 | Effect 1 7 |
|  | 074-075 | Effect 1 8 |
|  | 076-077 | Effect 1 9 |
|  | 078-079 | Effect 20 |
|  | 080-081 | Effect 21 |
|  | 082-083 | Effect 22 |
|  | 084-085 | Effect 23 |
|  | 086-087 | Effect 24 |
|  | 088-089 | Effect 25 |
|  | 090-091 | Effect 26 |
|  | 092-093 | Effect 27 |
|  | 094-095 | Effect 28 |
|  | 096-097 | Effect 29 |
|  | 098-033 | Effect 30 |
|  | 100-101 | Effect 31 |
|  | 102-103 | Effect 32 |
|  | 104-105 | Effect 33 |
|  | 106-107 | Effect 34 |
|  | 108-109 | Effect 35 |
|  | 110-111 | Effect 36 |
|  | 112-113 | Effect 37 |
|  | 114-115 | Effect 38 |
|  | 116-117 | Effect 39 |
|  | 118-119 | Effect 40 |
|  | 120-121 | Effect 41 |
|  | 122-123 | Effect 42 |
|  | 124-125 | Effect 43 |
|  | 126-127 | Effect 44 |
|  | 128-129 | Effect 45 |
|  | 130-131 | Effect 46 |
|  | 132-133 | Effect 47 |
|  | 134-135 | Effect 48 |
|  | 136-137 | Effect 49 |
|  | 138-139 | Effect 50 |
|  | 140-141 | Effect 51 |
|  | 142-143 | Effect 52 |
|  | 144-145 | Effect 53 |
|  | 146-147 | Effect 54 |
|  | 148-149 | Effect 55 |
|  | 150-151 | Effect 56 |
|  | 152-153 | Effect 57 |
|  | 154-155 | Effect 58 |
|  | 156-157 | Effect 59 |
|  | 158-159 | Effect 60 |
|  | 160-161 | Effect 61 |
|  | 162-163 | Effect 62 |
|  | 164-165 | Effect 63 |
|  | 166-167 | Effect 64 |
|  | 168-169 | Effect 65 |
|  | 170-171 | Effect 66 |
|  | 172-173 | Effect 67 |
|  | 174-175 | Effect 68 |
|  | 176-177 | Effect 69 |
|  | 178-179 | Effect 70 |
|  | 180-181 | Effect 71 |
|  | 182-183 | Effect 72 |
|  | 184-185 | Effect 73 |
|  | 186-187 | Effect 74 |
|  | 188-189 | Effect 75 |
|  | 190-191 | Effect 76 |
|  | 192-193 | Effect 77 |
|  | 194-195 | Effect 78 |
|  | 196-197 | Effect 79 |
|  | 198-199 | Effect 80 |
|  | 200-201 | Effect 81 |
|  | 202-203 | Effect 82 |
|  | 204-205 | Effect 83 |
|  | 206-207 | Effect 84 |
|  | 208-209 | Effect 85 |
|  | 210-211 | Effect 86 |
|  | 212-213 | Effect 87 |
|  | 214-215 | Effect 88 |
|  | 216-217 | Effect 89 |
|  | 218-219 | Effect 90 |
|  | 220-221 | Effect 91 |
|  | 222-223 | Effect 92 |
|  | 224-225 | Effect 93 |
|  | 226-227 | Effect 94 |
|  | 228-229 | Effect 95 |
|  | 230-231 | Effect 96 |
|  | 232-233 | Effect 97 |
|  | 234-235 | Effect 98 |
|  | 236-237 | Effect 99 |
|  | 238-239 | Effect 100 |
|  | 240-241 | Effect 101 |
|  | 242-243 | Effect 102 |
|  | 244-245 | Effect 103 |
|  | 246-247 | Effect 104 |
|  | 248-249 | Effect 105 |
|  | 250-251 | Effect 106 |
|  | 252-253 | Effect 107 |
|  | 254-255 | Effect 108 |
| 7 | 000-031 | Color option 1 |
|  | 032-063 | Color option 2 |
|  | 064-095 | Color option 3 |
|  | 096-127 | Color option 4 |
|  | 128-159 | Color option 5 |
|  | 160-191 | Color option 6 |
|  | 192-223 | Color option 7 |
|  | 224-255 | Color option 8 |
| 8 | 000-255 | Speed |

3 channels mode instruction

| Channel | Data | Function |
| --- | --- | --- |
| 1 | 000-255 | Red dimmer |
| 2 | 000-255 | Green dimmer |
| 3 | 000-255 | Blue dimmer |

四、DMX-512 control connection

3-pins DMX Out                       3-pins DMX In

(Female)                              (Male)

1 Earth          2 DMX-        3 DMX+

五、DMX address setting

Each fixture must set a specific starting address. When receiving signal transmission, fixture will receive channel control signals which from the starting address.

- According to different requirements or easy to use, the user can set many value fixtures to the same address, also can set up a separate address for every device.

- If you set the same address,all the devices will receive from DMX channels signal. All connecting fixtures are jointly controlled, controller can't separate control a device.

- If you set a different address, each device will be separate from its particular set the starting address and begin accepting DMX512 control signal, so that more convenient separate control one device. Using this method must be based on channel quantity of device to determine a starting address.

If led light has 8 channels.you should set the starting address of the first unit to 1, the second unit to 9(8 + 1), the third unit to 17 (8+ 9), and so on.

Construction and Materials

Ringing

1 Mounting bracket

2 Hanging bracket

- The applicable temperature for the lighting is between -25°C to 40°C. Do not use the lighting under or above the temperature.

- The installation of the effect has to be built and constructed in a way that it can hold 10 times the weight for 1 hour without any harming deformation.

- The installation must always be secured with a secondary safety attachment, e.g. an appropriate safety rope.Never stand directly below the device when mounting, removing or servicing the fixture.

- The operator has to make sure the safety relating and machine technical installations are approved 
by an expert before taking the device into operation for the first time.

- These installations have to be approved by a skilled person once a year.

- M8 screws on the mounting bracket which for fixed between hanging bracket’s ￠12.5mm hole.

- Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Wires connection must be professional person!

CAUTION

Warranty Card

We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeasure event. Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：