USER   MANUAL

280W PRISM KING BEAM MOVING HEAD LIGHT

Please read and keep this manual carefully for further use!

Thank you for buying our professional Beam moving head.

This user manual includes important information for installation and operation, so please read this user manual carefully before installation and operation, and keep this user manual well for further use.

In order to install , operate, and maintain the lighting safety and correctly. We suggest that the installation and operation should be done by the verified technician and follow the instruction strictly.

Spare part includes:

| Name | QTY | Unit |
| --- | --- | --- |
| Fixture | 1 | PC |
| User manual | 1 | PC |
| Power cable | 1 | PC |
| bracket | 2 | PCS |

Please check carefully that there is no damage caused by transportation. Should there be any, consult your local dealer.

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Silkscreen symbol on light

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Installation

► This fixture is not suitable for direct installation on combustible material surface, please keep the fixture surface at least 0.5m from any combustible material.

► If you use the quick lock cam in hanging up the fixture, please make sure the quick lock fasteners turned in the quick lock holes correctly.

►The applicable temperature for the fixture is between -25°C to 45°C. Do not use the lighting under or above this temperature range.

►The truss for hanging the fixture should be able to hold 10 times weight of this fixture and no deformation after 1 hour hang.

►Please don’t stand under the fixture when install, uninstall, move or repair the fixture.

►Please ensure the light is connected correctly and invite professional technician to check and confirm the electrical data before installation.

►Please invite professional technician to check the fixture and installation each year.

Rigging：

Clamp

Bracket

Safety rope

Quick lock

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Fix the clamp on truss or other fixture bracket.

Pull the safety rope through the holes on the bottom of the base and fasten it on the truss or other fixing point.

Note:This step is quite important to ensure that fixture will not drop down for the clamp damage.

Product specification

1. Voltage：100-240V，50/60HZ；

2. Power Consumption：300W；

3. average lifetime:2000hours；   Color Temp: 8000K；

4.Pan：540°(16bit) electronic error correction；

5.Tilt：270° (16bit) electronic error correction；

6.Color wheel:14 colors+open

7.Gobo wheel:13gobos+open; including 3 mirror gobos

8.Prism:5 prism effects

9.Frost and 6-color wheel effect,electronic focus

10.  Electrical dimming：0%~100%；support strobe at various speeds, strobe color marco

11.  Beam angle：1.8º

12. Over-heat protection；Display：LCD

13. IP rate:IP20；

14.Product size：327*268*490mm；

15.outer carton size: 405*370*53mm；

16.N.W：14.5KG；

Display and operation

| Touch button | Function |
| --- | --- |
| Up | Option, Edit |
| Left | Option, Edit |
| Right | Option, Edit |
| OK | Enter to edit or confirm |

Sub-menu operation mainly by touch, if using by Roller, operation instruction as below

<Up> : turn up;  <Down> ：turn down;  <OK>   ： enter or confirm

Menu operation

MAIL

MENU ----Setting       ----running mode

----DMX setting

----DMX channel mode

----sound sensitivity adjustment -2/+2

----X reverse

----Y reverse

----Y.X exchange

---- XY encode

---- RDM

----DMX lost

----display protection

----lamp ON

----factory default

---manual

---system       ----DMX channel value monitor

----system error record

----software version

----date

----turn on time

---senior        ----sensor monitoring

----reset correction

----gobo, color correction

---reset         ----all motors reset

--lamp

--English/Chinese

--display reverse

Manual operation

Press “OK” button to edit, choose digit at hundred place, choose UP or DOWN button to change the value, press OK to confirm.

System info

| Option | Details |
| --- | --- |
| software version | the current software version |
| DMX channel value | enter the sub-menu, show channel value or percentage |
| system error record | if the ERROR indicator is ON, it means the fixture has operation error, can enter the sub-menu to check and cancel the record |
| Error | Details |
| motor reset failure，serial port error | Driving PCB no feedback. the connection between display PCB and driving PCB has problem, or driving PCB problem |
| Pan reset error | Pan photo-electrical switch or Pan motor problem |
| Tilt reset error | Tilt photo-electrical switch or Tilt motor problem |
| Pan Hall error | Pan Hall error |
| Tilt Hall error | Tilt Hall error |
| color wheel reset failure | color wheel hall or color wheel motor problem |
| gobo wheel reset failure | gobo wheel hall or gobo wheel motor problem |
| focus reset failure | focus hall or focus motor error |
| lamp control failure | lamp ON/OFF error, lamp ballast or lamp have problem |

Senior

A password is set up to prevent from wrong operation by unprofessional people. Default password is “******”. press “OK” for password verification.

| Option | Details |
| --- | --- |
| reset | Enter the sub-menu, adjust the Pan/Tilt motor's reset position, the adjustment range is -128~+127，+0 means no adjustment |
| sensor monitoring | monitor the status of all sensor and Hall switch |

Gobo wheel

White 5

color sequence

Six-color effect

18 DMX channel

| 18CH | Function | Value | Details | Remarks |
| --- | --- | --- | --- | --- |
| CH1 | color wheel | 255 | high speed rotation | gobo flowing |
|  |  | …… | …… |  |
|  |  | 191 | low speed rotation |  |
|  |  | 190 | low speed rotation |  |
|  |  | …… | …… |  |
|  |  | 150 | high speed rotation |  |
|  |  | 145-149 | color 14+ white |  |
|  |  | 140 | color 14 |  |
|  |  | 135 | color 13 +color 14 | the value is 5N |
|  |  | 130 | color 13 |  |
|  |  | 125 | color 12 +color 13 |  |
|  |  | 120 | color 12 |  |
|  |  | 115 | color 11 +color 12 |  |
|  |  | 110 | color 11 |  |
|  |  | 105 | color 10 +color 11 |  |
|  |  | 100 | color 10 |  |
|  |  | 95 | color 9 +color 10 |  |
|  |  | 90 | color 9 |  |
|  |  | 85 | color 8 +color 9 |  |
|  |  | 80 | color 8 |  |
|  |  | 75 | color 7 +color 8 |  |
|  |  | 70 | color 7 |  |
|  |  | 65 | color 6 +color 7 |  |
|  |  | 60 | color 6 |  |
|  |  | 55 | color 5 +color 6 |  |
|  |  | 50 | color 5 |  |
|  |  | 45 | color 4 +color 5 |  |
|  |  | 40 | color 4 |  |
|  |  | 35 | color 3 +color 4 |  |
|  |  | 30 | color 3 |  |
|  |  | 25 | color 2 +color 3 |  |
|  |  | 20 | color 2 |  |
|  |  | 15 | color 1 +color 2 |  |
|  |  | 10 | color 1 |  |
|  |  | 5 | white +color 1 |  |
|  |  | 0 | white |  |
| CH2 | strobe | 252-255 | shutter open | controlled by dimmer channel |
|  |  | 239-251 | High speed random strobe |  |
|  |  | 226-238 | middle speed random strobe |  |
|  |  | 213-225 | low speed random strobe |  |
|  |  | 208-212 | shutter open | controlled by dimmer channel |
|  |  | 207 | Pulse strobe |  |
|  |  | …… | …… |  |
|  |  | 108 | Pulse strobe |  |
|  |  | 104-107 | shutter open | controlled by dimmer channel |
|  |  | 103 | strobe |  |
|  |  | …… | …… |  |
|  |  | 4 | strobe |  |
|  |  | 0-3 | shutter off |  |
| CH3 | dimmer | 255 | dimmer 100% |  |
|  |  | …… | …… |  |
|  |  | 0 | dimmer 0% |  |
| CH4 | gobo wheel | 255 | gobo 13 shake at high speed | the value is 5N |
|  |  | …… | …… |  |
|  |  | 251 | gobo 13 shake at low speed |  |
|  |  | …… | ……（gobo 2 to gobo 12） |  |
|  |  | 195 | gobo 1 shake at high speed |  |
|  |  | …… | …… |  |
|  |  | 191 | gobo 1 shake at low speed |  |
|  |  | 190 | high speed rotation（clockwise） |  |
|  |  | …… | …… |  |
|  |  | 135 | low speed rotation（clockwise） |  |
|  |  | 130-134 | stop |  |
|  |  | 129 | low speed rotation（anticlockwise） |  |
|  |  | …… | …… |  |
|  |  | 70 | high speed rotation（anticlockwise） |  |
|  |  | 65-69 | gobo 13 |  |
|  |  | 60 | gobo 12 |  |
|  |  | 55 | gobo 11 |  |
|  |  | 50 | gobo 10 |  |
|  |  | 45 | gobo 9 |  |
|  |  | 40 | gobo 8 |  |
|  |  | 35 | gobo 7 |  |
|  |  | 30 | gobo 6 |  |
|  |  | 25 | gobo 5 |  |
|  |  | 20 | gobo 4 |  |
|  |  | 15 | gobo 3 |  |
|  |  | 10 | gobo 2 |  |
|  |  | 5 | gobo 1 |  |
|  |  | 0 | white |  |
| CH5 | prism 1 | 200-255 | prism 2 |  |
|  |  | 100-199 | prism 1 |  |
|  |  | 0-99 | no effect |  |
| CH6 | prism 1 rotation | 255 | high speed rotation（clockwise） |  |
|  |  | …… | …… |  |
|  |  | 193 | low speed rotation（clockwise） |  |
|  |  | 191-192 | stop |  |
|  |  | 190 | low speed rotation（anticlockwise） |  |
|  |  | …… | …… |  |
|  |  | 127 | high speed rotation（anticlockwise） |  |
|  |  | 0-127 | adjust angle |  |
| CH7 | prism 12 | 200-255 | prism 5 |  |
|  |  | 101-199 | prism 4 |  |
|  |  | 10-100 | prism 3 |  |
|  |  | 0-9 | no effect |  |
| CH8 | prism 2 rotation | 255 | high speed rotation（clockwise） |  |
|  |  | …… | …… |  |
|  |  | 193 | low speed rotation（clockwise） |  |
|  |  | 191-192 | stop |  |
|  |  | 190 | low speed rotation（anticlockwise） |  |
|  |  | …… | …… |  |
|  |  | 127 | high speed rotation（anticlockwise） |  |
|  |  | 0-127 | adjust angle |  |
| CH9 | six color effect | 128-255 | six color effect |  |
|  |  | 0-127 | no six color effect |  |
| CH10 | focus | 255 | focus 100% |  |
|  |  | …… | …… |  |
|  |  | 0 | focus 0% |  |
| CH11 | focus fine | 0-255 | focus fine |  |
| CH12 | pan | 0-255 | 0-540 degree |  |
| CH13 | pan fine | 0-255 | 0-1 degree |  |
| CH14 | tilt | 0-255 | 0-270 degree |  |
| CH15 | tilt fine | 0-255 | 0-1 degree |  |
| CH16 | XY speed | 0-255 | from fast to slow |  |
| CH17 | frost | 128-255 | frost |  |
|  |  | 0-127 | no frost |  |
| CH18 | lamp control and reset | 245-250 | all motors reset | lamp ON/OFF after 5 seconds |
|  |  | 211-244 | no function |  |
|  |  | 200-210 | lamp on |  |
|  |  | 111-199 | no function |  |
|  |  | 100-110 | lamp off |  |
|  |  | 0-99 | no function |  |

DMX-512 connection

- Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.then connect the DMX signal to the first fixture to the 2nd fixture, after all fixtures are connected,

- The signal cable should be two - core with screened cable with XLR input and output connectors. Please refer to the diagram below

DMX terminator connection

If there are many lines or lights, it is recommended to use a DMX terminator to prevent DMX signal corruption, the DMX terminator is an simple XLR plug with a 120 Ω resistor connected between pins 2 and 3,which is then plugged into the output XLR socket of the last fixture in the chain.

3-Pins connection

Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Wires connection must be done professional person!

CAUTION

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

Common issues instruction

| Issues | Solve instruction |
| --- | --- |
| No power | Check power’s voltage |
| When power in, output light but the device cannot accept control signal | Check address setting and the control DMX cable connection well or not |
| The effect have light shadow | Check electronic focus DMX channel value, does suitable distance |
| Power down | The lamp maybe arrived end of using life, should change new lamp, and also check light source cleaning or not |

Note:  the maintenance should be done by the verified technician.